周报 — Unitree Go2 平地行走任务迁移

任务概述

将 IsaacLab 中 Unitree Go2 平地行走任务（Isaac-Velocity-Flat-Unitree-Go2-v0）迁移至 Motrix 框架，包括环境配置参数、奖励函数设计以及动作空间定义。

迁移对应关系

| IsaacLab | Motrix |
| --- | --- |
| flat_env_cfg.py + rough_env_cfg.py | _cfg.py |

walk_np.py 里面的内容——观测空间构成、奖励函数实现、动作计算逻辑——这些本身就是对应 IsaacLab 环境实现的结果，所以没有进行更改。

参数迁移情况

直接对应迁移的参数

- action_scale：0.25

- tracking_lin_vel 权重：1.5

- tracking_ang_vel 权重：0.75

- orientation 惩罚权重：-2.5

- feet_air_time 权重：0.25

- dof_acc 惩罚权重：-2.5e-7

有意调整的参数

- `torques` 惩罚：IsaacLab 中为 -0.0002，Motrix 中设为 0。原因是两个框架对力矩的计算方式不同，且 flat 任务中关闭该项对训练结果无显著影响，经实践验证。

- 速度指令范围：IsaacLab 中 x 方向最大 2.0 m/s，角速度最大 π rad/s；迁移版统一收窄为三轴对称的 [-1.0, 1.0]，训练分布更保守均匀。

关键问题发现与修正过程

1. action_rate 参数

在对照两版 Motrix 代码时发现，action_rate 惩罚权重从原版的 -0.001 变为了 -0.01，放大了 10 倍。是迁移过程中引入的无意改动，并非来自 IsaacLab 原始配置。经实际训练验证，该改动对结果无负面影响，予以保留。

2. action space 定义错误（已修正）

这是迁移中唯一的实质性错误。

问题根源：原版 Motrix 的设计是以 actuator_ctrl_limits（实际关节角度物理限位）作为动作空间边界，配合较小的 action_scale=0.05，两者是配套设计。迁移时只将 action_scale 改为 0.25 对齐 IsaacLab，但未同步修改动作空间定义。

验证问题：

加上print("actuator_ctrl_limits:", self._model.actuator_ctrl_limits)

测试脚本：

from motrix_envs.locomotion.go2 import walk_np  # 触发注册

from motrix_envs.locomotion.go2.cfg import Go2WalkNpEnvCfg

from motrix_envs.locomotion.go2.walk_np import Go2WalkTask

cfg = Go2WalkNpEnvCfg()

env = Go2WalkTask(cfg, num_envs=1)

测试结果：

12个执行器按腿重复4次，每条腿三个关节的范围是：

| 关节 | min | max |
| --- | --- | --- |
| hip | -0.9472 | 0.9472 |
| thigh | -1.4 | 2.5 |
| calf | -2.6227 | -0.8478 |

单位是弧度，这是​实际物理关节角度的限位​。

Calf最能说明问题：范围是 [-2.62, -0.85]，全是负数，因为 Go2 的小腿在物理上确实永远是向后弯的。这作为关节角度是合理的

但是IsaacLab 的 action 是无实际意义的归一化信号 [-1, 1]，由 action_scale 换算成关节角度。

Motrix 原版的 action 直接就是关节角度本身，范围就是关节的物理限位。

实际影响：打印 actuator_ctrl_limits 后发现其范围并非 [-1, 1]，例如 calf 关节的范围为 [-2.62, -0.85]，全为负值。这导致：

- 与 IsaacLab 的 action ∈ [-1,1] × 0.25 语义不等价

- calf 关节动作空间异常，小腿抬起空间极为受限

训练仍能成功，是因为 RL 策略在实践中自适应地输出了较小的值，绕开了该问题，属于"歪打正着"。

修正方案：将 _init_action_space 中的动作空间边界改为固定的 [-1, 1]，使其与 IsaacLab 完全对齐。

# 修正前：以物理限位作为边界    
    gym.spaces.Box(actuator_ctrl_limits[0], actuator_ctrl_limits[1], ...)    
         
    # 修正后：固定归一化范围    
    gym.spaces.Box(-np.ones(n), np.ones(n), ...)

原来的函数：

修改过后：

训练结果

修正后训练可以正常达成平地行走任务，机器人能够跟踪线速度和角速度指令，步态稳定。

附录

Motrix的代码cfg.py

# Copyright (C) 2020-2025 Motphys Technology Co., Ltd. All Rights Reserved.

#

# Licensed under the Apache License, Version 2.0 (the "License");

# you may not use this file except in compliance with the License.

# You may obtain a copy of the License at

#

#     http://www.apache.org/licenses/LICENSE-2.0

#

# Unless required by applicable law or agreed to in writing, software

# distributed under the License is distributed on an "AS IS" BASIS,

# WITHOUT WARRANTIES OR CONDITIONS OF ANY KIND, either express or implied.

# See the License for the specific language governing permissions and

# limitations under the License.

# ==============================================================================

import os

from dataclasses import dataclass, field

from motrix_envs import registry

from motrix_envs.base import EnvCfg

model_file = os.path.dirname(__file__) + "/xmls/scene_flat.xml"

@dataclass

class NoiseConfig:

level: float = 1.0

scale_joint_angle: float = 0.03

scale_joint_vel: float = 1.5

scale_gyro: float = 0.2

scale_gravity: float = 0.05

scale_linvel: float = 0.1

@dataclass

class ControlConfig:

# action scale: target angle = actionScale * action + defaultAngle

action_scale = 0.05

@dataclass

class InitState:

# the initial position of the robot in the world frame

pos = [0.0, 0.0, 0.42]  # 0.278

# the default angles for all joints. key = joint name, value = target angle [rad]

default_joint_angles = {

"FL_hip": 0.1,  # [rad]

"FL_thigh": 0.9,  # [rad]

"FL_calf": -1.8,  # [rad]

"FR_hip": -0.1,  # [rad]

"FR_thigh": 0.9,  # [rad]

"FR_calf": -1.8,  # [rad]

"RL_hip": 0.1,  # [rad]

"RL_thigh": 0.9,  # [rad]

"RL_calf": -1.8,  # [rad]

"RR_hip": -0.1,  # [rad]

"RR_thigh": 0.9,  # [rad]

"RR_calf": -1.8,  # [rad]

}

@dataclass

class Commands:

vel_limit = [

[-2.0, -1.0, -3.1416],  # min: vel_x [m/s], vel_y [m/s], ang_vel [rad/s]

[2.0, 1.0, 3.1416],  # max

]

@dataclass

class Normalization:

lin_vel = 2

ang_vel = 0.25

dof_pos = 1

dof_vel = 0.05

@dataclass

class Asset:

body_name = "base"

foot_name = "foot"

penalize_contacts_on = ["thigh", "calf"]

terminate_after_contacts_on = [

"base_collision_0",

"base_collision_1",

"base_collision_2",

"fl_hip_0",

"fr_hip_0",

"rl_hip_0",

"rr_hip_0",

]

ground = "floor"

@dataclass

class Sensor:

local_linvel = "local_linvel"

gyro = "gyro"

@dataclass

class RewardConfig:

scales: dict[str, float] = field(

default_factory=lambda: {

"termination": -0.0,

"tracking_lin_vel": 1.0,

"tracking_ang_vel": 0.5,

"lin_vel_z": -2.0,

"ang_vel_xy": -0.05,

"orientation": -0.0,

"torques": -0.00001,

"dof_vel": -0.0,

"dof_acc": -2.5e-7,

"base_height": -0.0,

"feet_air_time": 1.0,

"collision": -1.0 * 0,

"feet_stumble": -0.0,

"action_rate": -0.001,

"stand_still": -0.0,

"hip_pos": -1,

"calf_pos": -0.3 * 0,

}

)

tracking_sigma: float = 0.25

max_foot_height: float = 0.1

@registry.envcfg("go2-flat-terrain-walk")

@dataclass

class Go2WalkNpEnvCfg(EnvCfg):

max_episode_seconds: float = 20.0

model_file: str = model_file

noise_config: NoiseConfig = field(default_factory=NoiseConfig)

control_config: ControlConfig = field(default_factory=ControlConfig)

reward_config: RewardConfig = field(default_factory=RewardConfig)

init_state: InitState = field(default_factory=InitState)

commands: Commands = field(default_factory=Commands)

normalization: Normalization = field(default_factory=Normalization)

asset: Asset = field(default_factory=Asset)

sensor: Sensor = field(default_factory=Sensor)

sim_dt: float = 0.01

ctrl_dt: float = 0.01

IsaacLab上的flat_env_cfg.py

# Copyright (c) 2022-2026, The Isaac Lab Project Developers (https://github.com/isaac-sim/IsaacLab/blob/main/CONTRIBUTORS.md).

# All rights reserved.

#

# SPDX-License-Identifier: BSD-3-Clause

from isaaclab.utils import configclass

from .rough_env_cfg import UnitreeGo2RoughEnvCfg

@configclass

class UnitreeGo2FlatEnvCfg(UnitreeGo2RoughEnvCfg):

def __post_init__(self):

# post init of parent

super().__post_init__()

# override rewards

self.rewards.flat_orientation_l2.weight = -2.5

self.rewards.feet_air_time.weight = 0.25

# change terrain to flat

self.scene.terrain.terrain_type = "plane"

self.scene.terrain.terrain_generator = None

# no height scan

self.scene.height_scanner = None

self.observations.policy.height_scan = None

# no terrain curriculum

self.curriculum.terrain_levels = None

class UnitreeGo2FlatEnvCfg_PLAY(UnitreeGo2FlatEnvCfg):

def __post_init__(self) -> None:

# post init of parent

super().__post_init__()

# make a smaller scene for play

self.scene.num_envs = 50

self.scene.env_spacing = 2.5

# disable randomization for play

self.observations.policy.enable_corruption = False

# remove random pushing event

self.events.base_external_force_torque = None

self.events.push_robot = None

IsaacLab上的rough_env_cfg.py

# Copyright (c) 2022-2026, The Isaac Lab Project Developers (https://github.com/isaac-sim/IsaacLab/blob/main/CONTRIBUTORS.md).

# All rights reserved.

#

# SPDX-License-Identifier: BSD-3-Clause

from isaaclab.utils import configclass

from .rough_env_cfg import UnitreeGo2RoughEnvCfg

@configclass

class UnitreeGo2FlatEnvCfg(UnitreeGo2RoughEnvCfg):

def __post_init__(self):

# post init of parent

super().__post_init__()

# override rewards

self.rewards.flat_orientation_l2.weight = -2.5

self.rewards.feet_air_time.weight = 0.25

# change terrain to flat

self.scene.terrain.terrain_type = "plane"

self.scene.terrain.terrain_generator = None

# no height scan

self.scene.height_scanner = None

self.observations.policy.height_scan = None

# no terrain curriculum

self.curriculum.terrain_levels = None

class UnitreeGo2FlatEnvCfg_PLAY(UnitreeGo2FlatEnvCfg):

def __post_init__(self) -> None:

# post init of parent

super().__post_init__()

# make a smaller scene for play

self.scene.num_envs = 50

self.scene.env_spacing = 2.5

# disable randomization for play

self.observations.policy.enable_corruption = False

# remove random pushing event

self.events.base_external_force_torque = None

self.events.push_robot = None